# Amplifier Classes and Filter Circuit Solutions

# Question i) Differentiate between Class-C and Class-D Amplifier

## Class-C Amplifier

A Class-C amplifier is a highly efficient non-linear amplifier with the following characteristics:

Conduction angle: The transistor conducts for less than 90° of the input cycle (typically 60° to 120°)

Biasing: The transistor is biased well into the cutoff region, with the bias voltage significantly below the turn-on threshold

Input signal requirement: Requires a large input signal swing to drive the transistor into conduction

Output: Produces highly distorted sinusoidal pulses that must be filtered by a tuned LC tank circuit

Efficiency: Extremely high efficiency (up to 80-90%) due to minimal overlap between voltage and current waveforms

Applications: Narrowband RF power amplifiers, transmitters operating at fixed frequencies

Harmonic content: High harmonic distortion; requires tuned load circuit to extract fundamental frequency

Linearity: Poor linearity; suitable only for constant-envelope signals (FM, CW)

## Class-D Amplifier

A Class-D amplifier is a switching amplifier with the following characteristics:

Conduction mode: The transistor operates as a switch, alternating between ON (saturation) and OFF (cutoff) states

Biasing: No DC bias; transistor is either fully conducting or completely off

Input signal: Uses pulse-width modulation (PWM) or pulse-density modulation to encode the analog signal

Output: Square wave switching signal at high frequency, then filtered to recover the audio/analog signal

Efficiency: Very high efficiency (90-98%) with minimal power dissipation in the switching transistors

Applications: Audio amplifiers, digital amplifiers, motor drivers, DC-DC converters

Harmonic content: Switching frequency and harmonics are removed by output filter, leaving clean output

Linearity: Excellent linearity; preserves signal fidelity over wide frequency range

## Comparison Table

| Parameter | Class-C | Class-D |
| --- | --- | --- |
| Conduction Angle | <90° (60°-120°) | 180° (switching mode) |
| Bias Condition | Below cutoff (highly negative) | Between cutoff and saturation |
| Operating Mode | Linear (partially conducting) | Switching mode (on/off) |
| Efficiency | 80-90% | 90-98% |
| Output Signal Quality | Highly distorted pulses | Clean after filtering |
| Linearity | Very poor (non-linear) | Excellent (linear) |
| Harmonic Content | High (needs tuned load) | Switched at fixed frequency |
| Tank Circuit | Required (tuned LC) | Not required (uses filter) |
| Suitable For | Constant-envelope RF signals | Audio, wideband signals |
| Load Impedance | Highly frequency dependent | Relatively frequency independent |
| Power Dissipation | Moderate | Very low |
| Applications | RF amplifiers, transmitters | Audio amplifiers, digital audio |

# Question ii) Filter Circuit Type and Working Procedure

## Filter Circuit Identification

Based on the circuit diagram provided, this is a

PASSIVE RC LOW-PASS FILTER

Circuit Components:

Two resistors (R₁ and R₂) connected in series

Two capacitors (C₁ and C₂) connected to ground

This configuration creates a 2nd-order (two-pole) RC low-pass filter

OpAmp buffer for isolation and impedance matching

## Working Procedure of the RC Low-Pass Filter

### 1. Signal Attenuation by Frequency

At low frequencies (f << f_c):

The capacitive reactance X_c = 1/(2πfC) is very high (capacitors act as open circuits)

Signal passes through the resistors with minimal attenuation

Output voltage ≈ Input voltage (gain ≈ 1 or 0 dB)

At high frequencies (f >> f_c):

The capacitive reactance X_c becomes very low (capacitors act as short circuits)

Capacitors C₁ and C₂ provide low-impedance paths to ground

Signal is heavily attenuated and shorted to ground

Output voltage → 0V (gain → -∞ dB)

### 2. Frequency Response

The cutoff frequency (-3dB frequency) is determined by:

f_c = 1 / (2π × R_eq × C_eq)

Where:

R_eq is the equivalent series resistance

C_eq is the equivalent capacitance

### 3. Phase Shift

At low frequencies: Phase shift ≈ 0°

At f_c (cutoff frequency): Phase shift = -45° per pole, so total = -90° for 2-pole filter

At high frequencies: Phase shift → -180° (for 2-pole filter)

### 4. Roll-off Rate

This is a 2nd-order filter (2 poles):

Roll-off rate = -40 dB/decade or -12 dB/octave

Signal amplitude decreases by factor of 100 for each 10× increase in frequency above f_c

### 5. Operating Procedure Step-by-Step

Step 1 - Input Signal Reception:

Analog signal (mix of DC, low-frequency, and high-frequency components) enters the filter

Step 2 - Frequency Separation:

Capacitor C₁ blocks DC component (capacitor is open to DC)

Low frequencies pass through R₁ and charge C₁

High frequencies are attenuated by the RC network

Step 3 - Second Stage Filtering:

Signal passes through R₂ and encounters C₂

C₂ provides additional frequency-dependent attenuation

The RC combination creates the 2nd-order response

Step 4 - Output Signal Buffering:

OpAmp acts as a voltage follower (unity-gain buffer)

High input impedance prevents loading effects

Low output impedance provides clean signal output

Step 5 - Output Signal Characteristics:

Only frequencies below f_c are significantly passed

Unwanted high-frequency noise and switching ripple are attenuated

Output is a smooth, filtered version of the input

## Practical Application

This filter is commonly used to:

Remove high-frequency switching noise from Class-D amplifier outputs

Smooth DC power supply outputs

Eliminate aliases in audio signal processing

Reduce electromagnetic interference (EMI) in power electronics

## Frequency Response Graph Interpretation

The graph provided shows:

Passband (flat response) at frequencies below f_c with gain ≈ 0 dB or A_v ≈ 1

Cutoff frequency f_c where gain drops to -3 dB (0.707 × maximum gain)

Stopband (attenuation region) at frequencies above f_c with -40 dB/decade roll-off

Transition region showing smooth slope from passband to stopband

# Summary of Filter Parameters

| Parameter | Description |
| --- | --- |
| Filter Type | 2nd-Order Passive RC Low-Pass Filter |
| Configuration | Two RC stages with OpAmp buffer |
| Cutoff Frequency | f_c = 1/(2π√(R₁R₂C₁C₂)) |
| Roll-off Rate | -40 dB/decade (2nd-order) |
| Phase Shift at f_c | -90° (for 2-pole filter) |
| High Frequency Behavior | Approaches -180° phase shift |
| Passband Gain | ≈ 0 dB (unity gain) |
| Stopband Attenuation | -40 dB/decade above f_c |
| DC Blocking | C₁ blocks DC component |
| Output Impedance | Low (due to OpAmp buffer) |