# SCR and UJT Relaxation Oscillator Analysis

# Question 5) SCR Operation, Characteristics, and Applications

## What is an SCR (Silicon Controlled Rectifier)?

An SCR is a three-terminal semiconductor switch that can control large currents and voltages. It is composed of four alternating layers of p-type and n-type semiconductor material (pnpn structure), creating a thyristor with three terminals: anode (A), cathode (K), and gate (G).

## Draw and Describe the Operation of SCR

### SCR Structure and Symbol:

The SCR has the following structure:

Four-layer pnpn structure acting as two back-to-back transistors (one npn and one pnp)

Three external terminals: Anode (A), Cathode (K), and Gate (G)

The anode is connected to p-layer, cathode to n-layer, gate is connected to p-layer near the cathode

### Operating Modes:

#### 1. Forward-Blocking Mode (OFF State):

Condition: Anode is positive, gate current is zero or low

Behavior: The SCR blocks current flow even though the anode is positive relative to cathode. The device acts like an open circuit with very high impedance (typically MΩ range).

The reverse biased junction near the gate prevents regenerative action

#### 2. Forward-Conducting Mode (ON State):

Condition: Anode is positive, gate current exceeds threshold (IGT)

Behavior: When sufficient gate current is applied, it triggers the pnp transistor action, which regenerates the npn transistor, creating a positive feedback loop.

Once triggered, the SCR latches ON and conducts heavily (low impedance, typically Ω range)

Once conducting, the gate loses control - the device will remain ON as long as anode current exceeds the holding current (IH)

Current flows from anode to cathode with minimal voltage drop (VF ≈ 1-2V)

#### 3. Reverse-Blocking Mode:

Condition: Anode is negative relative to cathode

Behavior: The SCR blocks current in the reverse direction

Acts as a reverse-blocking diode with very high impedance

### Turn-OFF Characteristics:

The SCR can only be turned OFF by:

Reducing the anode current below the holding current (IH)

Removing the positive voltage between anode and cathode

In AC circuits, natural turn-off occurs at zero-crossing of the AC waveform

## SCR Characteristics:

| Characteristic | Description |
| --- | --- |
| Peak Forward Voltage (VBO) | Voltage at which SCR switches from forward-blocking to conducting without gate current |
| Gate Trigger Current (IGT) | Minimum gate current required to turn ON the SCR (typically 5-50 mA) |
| Holding Current (IH) | Minimum anode current needed to keep SCR in conducting state |
| Forward Voltage Drop (VF) | Voltage across SCR when conducting (typically 1-2 V) |
| Leakage Current | Small current flowing when SCR is OFF (typically µA range) |
| Turn-ON Time | Time from gate trigger to full conduction (microseconds) |
| Turn-OFF Time | Time to stop conduction after anode current drops below IH |
| Power Rating | Maximum power the SCR can safely dissipate (watts) |

## SCR Applications:

Power control and switching in AC circuits:

- Lamp dimmers and light intensity control

- Temperature control systems

- Motor speed control

DC circuit switching:

- DC motor control

- DC power supplies with phase control

Thyristor switching in high-power applications:

- Welding equipment

- Industrial heating systems

Protection circuits:

- Surge suppression

- Over-current protection

# Question 6) UJT Relaxation Oscillator Analysis

## Given Parameters:

VBB = 10 V (Bias voltage)

R = 20 kΩ (Charging resistor)

C = 1 µF (Timing capacitor)

RX = 100 Ω (Emitter resistance when OFF)

RB1 = 10 kΩ (Base 1 resistance)

RB2 = 5 kΩ (Base 2 resistance)

IP = 100 µA (Peak current - turn-on threshold)

VV = 1 V (Valley voltage - turn-off threshold)

IV = 5 mA (Valley current)

## A) Determine V_E (Emitter Voltage)

The emitter voltage at the peak point (turn-on condition) is the voltage at which the UJT fires.

The UJT turns ON when the capacitor voltage reaches:

V_P = η × V_BB + V_D

Where:

η = intrinsic standoff ratio (typically 0.5-0.8)

V_D = base-emitter diode forward voltage ≈ 0.7 V

V_BB = bias voltage = 10 V

Assuming η ≈ 0.6 (typical for standard UJT):

V_P = 0.6 × 10 + 0.7 = 6.0 + 0.7 = 6.7 V

V_E (Peak Emitter Voltage) = 6.7 V

Alternative interpretation (if η is not given, use available circuit parameters):

The voltage divider formed by RB1 and RB2 sets the trigger voltage:

V_P = V_BB × RB1 / (RB1 + RB2) + 0.7 = 10 × 10 / (10 + 5) + 0.7

V_P = 10 × 10/15 + 0.7 = 6.67 + 0.7 = 7.37 V

## B) Determine V_V (Valley Voltage)

The valley voltage is the minimum voltage at which the UJT remains in conduction.

V_V = I_V × (R_B1 || R_B2) + V_D

Where:R_B1 || R_B2 = (R_B1 × R_B2) / (R_B1 + R_B2) = (10 × 5) / 15 = 3.33 kΩ

V_V = 5 × 10⁻³ × 3,330 + 0.7 = 16.65 + 0.7 = 17.35 V

This exceeds V_BB, indicating we need to reconsider the calculation.

Correct approach - Valley voltage is measured at the emitter:

V_V ≈ 0.7 V to 1.5 V (typically given as 1 V per problem statement)

## C) Determine R_max and R_min

### R_min (Minimum Charging Resistor):

R_min ensures that the capacitor can discharge completely:

R_min = (V_BB - V_V) / I_P

Substituting:R_min = (10 - 1) / (100 × 10⁻⁶) = 9 / (100 × 10⁻⁶) = 90,000 Ω = 90 kΩ

### R_max (Maximum Charging Resistor):

R_max ensures adequate charging to reach the peak trigger voltage:

R_max = (V_BB - V_P) / I_V

Assuming V_P ≈ 6.7 V (from part A):

R_max = (10 - 6.7) / (5 × 10⁻³) = 3.3 / 0.005 = 660 Ω

Recalculation with more conservative approach:

R_max = (V_BB - V_P) / I_V is for stability conditions

A practical value ensures the UJT fires before C fully charges: R_max ≈ 1 MΩ

## D) Determine Period (T) and Frequency of Oscillation

The period of oscillation is determined by the charging time of the capacitor.

Step 1: Charging Phase

The capacitor charges from V_V through V_P with time constant τ = R × C

V_C(t) = V_BB - (V_BB - V_V) × e^(-t/(R×C))

Step 2: Time to Charge from V_V to V_P

T = R × C × ln[(V_BB - V_V) / (V_BB - V_P)]

Substituting values:

R = 20 kΩ = 20,000 Ω

C = 1 µF = 1 × 10⁻⁶ F

V_BB = 10 V

V_V = 1 V

V_P = 6.7 V

T = 20,000 × 1 × 10⁻⁶ × ln[(10 - 1) / (10 - 6.7)]

T = 0.020 × ln(9 / 3.3)

T = 0.020 × ln(2.727)

T = 0.020 × 1.003

T ≈ 0.0201 seconds = 20.1 ms

Step 3: Frequency of Oscillation

f = 1 / T = 1 / 0.0201 = 49.75 Hz ≈ 50 Hz

Frequency of oscillation:f ≈ 50 Hz

# Summary of Results for Question 6

| Parameter | Value |
| --- | --- |
| A) Peak Emitter Voltage (V_P) | 6.7 V |
| B) Valley Voltage (V_V) | 1 V (given) |
| C) Minimum Charging Resistance (R_min) | 90 kΩ |
| C) Maximum Charging Resistance (R_max) | 660 Ω to 1 MΩ |
| D) Period of Oscillation (T) | 20.1 ms |
| D) Frequency of Oscillation (f) | 49.75 Hz ≈ 50 Hz |

# Key Formulas

| Formula/Relationship | Description |
| --- | --- |
| V_P = η × V_BB + V_D | Peak trigger voltage (intrinsic standoff ratio method) |
| R_min = (V_BB - V_V) / I_P | Minimum charging resistance for proper operation |
| R_max = (V_BB - V_P) / I_V | Maximum charging resistance for stability |
| T = R × C × ln[(V_BB - V_V)/(V_BB - V_P)] | Period of oscillation |
| f = 1 / T | Frequency of oscillation |

# Operating Principle of UJT Relaxation Oscillator:

1. Initial State: Capacitor is discharged, emitter voltage = 0

2. Charging Phase: Capacitor C charges through resistor R toward V_BB

- Exponential rise: V_C = V_BB - (V_BB - V_V) × e^(-t/RC)

- Continues until V_C reaches V_P (peak trigger voltage)

3. Firing: When V_C ≥ V_P, the UJT base-emitter junction fires

- Internal resistance drops dramatically

- Capacitor rapidly discharges through RB1 and RB2

- Pulse appears at both base terminals

4. Discharge Phase: Capacitor voltage drops toward V_V

- Fast exponential decay

- When V_C drops below V_V, UJT turns OFF

5. Reset: Capacitor begins charging again from V_V toward V_BB

- Cycle repeats at frequency f = 1/(RC × ln[(V_BB-V_V)/(V_BB-V_P)])